3. Static Structural Analysis of Fixed Beam

Reg. No:

Date:

Aim

To perform static structural analysis of a fixed beam with the application of distributed load and point load separately at its top surface.

Software Required

ANSYS Workbench (Student / Academic Version)

Problem Statement

Analyse the deformation and stress distribution of a fixed beam subjected to an external load using ANSYS Workbench.

Theory (Brief)

Static structural analysis is used to determine stresses, strains and deformation caused by applied loads.

Procedure

- Open ANSYS Workbench.

- Drag and drop Static Structural system into the Project Schematic.

- Double click Geometry and create a rectangular beam using SpaceClaim/Design Modeler.

- Set beam dimensions: Length = 500 mm, Width = 20 mm, Height = 10 mm.

- Close and save geometry.

- Open Model and assign Structural Steel as material.

- Generate mesh using default settings.

- Apply fixed boundary condition at the ends of the beam

- Apply distributed load of 5000 N / m-length downward at the top surface.

- Solve the model.

- Insert Total Deformation and Equivalent (von-Mises) Stress, save result pictures and note down the results.

- Change Force to 2000 N point load at the centre, resolve and measure maximum deformation and von-Mises Stresses. Record the new results.

Results

The maximum deformation and maximum bending stress occur at the centre of the beam during case-1 (distributed load) and case-2 (point load at the centre)

Total deformation

The value of maximum deformation for different cases:

Contour plots:

von-Mises Stresses

The value of maximum stress for different cases:

Contour plot:

Conclusion

Static structural analysis of fixed beam is successfully performed, and deformation & stress distribution are visualized.