Experiment 1

Introduction to ANSYS Workbench

| Name Sam Blesson S | Reg. No. 2023111073 |
| --- | --- |
| Date 25-08-2026 | Software ANSYS Workbench (Student) |

# Aim

To introduce the ANSYS Workbench interface and perform a basic static structural analysis of a cantilever beam.

# Software Required

ANSYS Workbench (Student / Academic Version)

# Problem Statement

Analyse the deformation and stress distribution of a cantilever beam subjected to an external load using ANSYS Workbench.

# Theory

Static structural analysis is used to determine the stresses, strains, and deformation caused by applied loads that do not vary with time.

# Procedure

- Open ANSYS Workbench.

- Drag and drop a Static Structural system into the Project Schematic.

- Double-click Geometry and create a rectangular beam using SpaceClaim.

- Set beam dimensions: Length = 200 mm, Width = 20 mm, Height = 20 mm.

- Close and save the geometry.

- Open Model and assign Structural Steel as the material.

- Generate the mesh using default settings.

- Apply a Fixed Support at one end of the beam.

- Apply a Force of 1000 N downward at the free end.

- Solve the model.

- Insert Total Deformation and Equivalent (von-Mises) Stress and note the results.

- Change the Force to 2000 N, re-solve, and measure the maximum deformation and von-Mises stress.

# Results

The maximum deformation occurs at the free end of the beam, and the maximum stress occurs near the fixed support.

## For 1000 N

Total Deformation

The value of maximum deformation is 0.37532 mm.

Contour plot:

Fig. 1 — Total Deformation, 1000 N

von-Mises Stress

The value of maximum stress is 73.005 MPa.

Contour plot:

Fig. 2 — Equivalent (von-Mises) Stress, 1000 N

## For 2000 N

Total Deformation

The value of maximum deformation is 0.75063 mm.

Contour plot:

Fig. 3 — Total Deformation, 2000 N

von-Mises Stress

The value of maximum stress is 146.01 MPa.

Contour plot:

Fig. 4 — Equivalent (von-Mises) Stress, 2000 N

# Summary of Results

| Applied Load | Max. Total Deformation | Max. von-Mises Stress |
| --- | --- | --- |
| 1000 N | 0.37532 mm | 73.005 MPa |
| 2000 N | 0.75063 mm | 146.01 MPa |

# Conclusion

ANSYS Workbench was successfully used to perform static structural analysis of a cantilever beam, and the resulting deformation and stress distributions were visualised for two load cases.