- Static Structural Analysis of Simply Supported Beam

Reg. No:

Date:

Aim

To perform static structural analysis of a simply supported beam with point load at the centre and distributed load at its top surface. To check the result variation while using fine mesh.

Software Required

ANSYS Workbench (Student / Academic Version)

Problem Statement

Analyse the deformation and stress distribution of a simply supported beam subjected to an external load using ANSYS Workbench.

Theory (Brief)

Static structural analysis is used to determine stresses, strains and deformation caused by applied loads that do not vary with time.

Procedure

- Open ANSYS Workbench.

- Drag and drop Static Structural system into the Project Schematic.

- Double click Geometry and create a rectangular beam using SpaceClaim.

- Set beam dimensions: Length = 200 mm, Width = 20 mm, Height = 20 mm.

- Close and save geometry.

- Open Model and assign Structural Steel as material.

- Generate mesh using default settings.

- Apply simply Support condition at the ends of the beam (Displacement constraint (Dy=0, Dz=0, Horizontal direction free)

- Apply Force of 2000 N downward at the top surface.

- Solve the model.

- Insert Total Deformation and Equivalent (von-Mises) Stress and note down the results.

- Change Force to 2000 N point load at the centre, resolve and measure maximum deformation and von-Mises Stresses.

- Change the mesh to fine mesh, repeat the analysis and take results for both the above cases,

Results

The maximum deformation and maximum bending stress occur at the centre of the beam.

Total deformation

The value of maximum deformation for different cases:

Contour plot:

von-Mises Stress

The value of maximum stress for different cases:

Contour plot:

Conclusion

Static structural analysis of simply supported beam is successfully performed, and deformation & stress distribution are visualized.